/* Program to turn Green LED(PF3) ON & OFF with a Delay of 1sec */

#include “tm4c123gh6pm.h”

int main()

{

SYSCTL_RCGCGPIO_R |= 0x20; // turn on bus clock for GPIOF

GPIO_PORTF_DIR_R      |= 0x08; //set GREEN pin as a digital output pin

GPIO_PORTF_DEN_R       |= 0x08;  // Enable PF3 pin as a digital pin

NVIC_ST_RELOAD_R = 15999999; // one second delay relaod value

NVIC_ST_CTRL_R = 5 ; // enable counter and select system bus clock

NVIC_ST_CURRENT_R  = 0; //initialize current value register

while (1)

{

if(NVIC_ST_CTRL_R & 0x10000)

{

GPIO_PORTF_DATA_R  ^= 8;

}

}

}